- TOGGLE LED WITH SOME DELAY USING PUSH PULL CONFIGURATION USING OUPUT PIN

#include "stm32f407xx.h"

void delay(void){

for(uint32_t i = 0; i < 5000000 ; i++);

}

int main(void){

GPIO_Handle_t Gpioled;

Gpioled.pGPIOx = GPIOD;  //pd12  led4 so port d

Gpioled.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

Gpioled.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

Gpioled.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

Gpioled.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_PP;  // push pull config

Gpioled.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD; // no enabling because already it is push pull

GPIO_PeriClockControl(GPIOD, ENABLE);  // enabling peripheral clock due to lot of peripherals are required to control

GPIO_Init(&Gpioled);

/* Toggle led */

while(1){

GPIO_ToggleOutputPin(GPIOD,GPIO_PIN_NO_12);

delay();

}

//GPIO_Init(&Gpioled);

return 0;

}

- LED TOGGLE USING OPEN DRAIN CONFIGURATION FOR THE OUTPUT PIN

Gpioled.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_OD;  // push pull config

Gpioled.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD; // no enabling because already it is push pull

For the above program the led will not blink.

OR

Gpioled.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_OD;  // push pull config

Gpioled.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_PIN_PU;

For the above program LED will blink with very low intensity

Another way to blink LED is Take jumper wire and make connections from PD12 to VCC 5volts on board connect through a resistor this can be done by GPIO_NO_PUPD

- LED TOGGLE USING INTERNAL BUTTON

#include "stm32f407xx.h"

#define HIGH 1

#define BTN_PRESSED HIGH

void delay(void){

for(uint32_t i = 0; i < 500000/2 ; i++);

}

int main(void){

GPIO_Handle_t Gpioled,GPIOBtn;

Gpioled.pGPIOx = GPIOD;  //pd12  led4 so port d

Gpioled.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

Gpioled.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

Gpioled.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

Gpioled.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_PP;  // push pull config

Gpioled.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD; // no enabling because already it is push pull

GPIO_PeriClockControl(GPIOD, ENABLE);  // enabling peripheral clock due to lot of peripherals are required to control

GPIO_Init(&Gpioled);

GPIOBtn.pGPIOx = GPIOA;  //PA0 B1 USER so A port 0 pin

GPIOBtn.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_0;

GPIOBtn.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_IN;

GPIOBtn.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

//GPIOBtn.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_OD;  does not apply because only when mode is output it is applicable

GPIOBtn.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD; // no enabling because already it is push pull

GPIO_PeriClockControl(GPIOA, ENABLE);  // enabling peripheral clock due to lot of peripherals are required to control

GPIO_Init(&GPIOBtn);

/* Toggle led */

while(1){

if(GPIO_ReadFromInputPin(GPIOA, GPIO_PIN_NO_0) == BTN_PRESSED)

{

delay();

GPIO_ToggleOutputPin(GPIOD,GPIO_PIN_NO_12);

}

}

//GPIO_Init(&Gpioled);

return 0;

}

- TOGLLE LED USING EXTERNAL BUTTON AND LED

#include "stm32f407xx.h"

#define HIGH 1

#define LOW  0

#define BTN_PRESSED HIGH    // LOW

void delay(void){

for(uint32_t i = 0; i < 500000/2 ; i++);

}

int main(void){

GPIO_Handle_t Gpioled,GPIOBtn;

Gpioled.pGPIOx = GPIOA;

Gpioled.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_8;

Gpioled.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

Gpioled.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

Gpioled.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_PP;  // push pull config

Gpioled.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD; // no enabling because already it is push pull

GPIO_PeriClockControl(GPIOA, ENABLE);  // enabling peripheral clock due to lot of peripherals are required to control

GPIO_Init(&Gpioled);

GPIOBtn.pGPIOx = GPIOB;

GPIOBtn.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GPIOBtn.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_IN;

GPIOBtn.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

//GPIOBtn.GPIO_PinConfig.GPIO_PinOPtype = GPIO_OP_TYPE_OD;  does not apply because only when mode is output it is applicable

GPIOBtn.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_PIN_PU; // Enabling push pull

GPIO_PeriClockControl(GPIOA, ENABLE);  // enabling peripheral clock due to lot of peripherals are required to control

GPIO_Init(&GPIOBtn);

/* Toggle led */

while(1){

if(GPIO_ReadFromInputPin(GPIOB, GPIO_PIN_NO_12) == BTN_PRESSED)

{

delay();

GPIO_ToggleOutputPin(GPIOA,GPIO_PIN_NO_8);

}

}

//GPIO_Init(&Gpioled);

return 0;

}